East West University

Department of Computer Science and Engineering

Course: CSE251 Electronic Circuits

Expt No: 03

Title: Study of Zener Diode.

Objectives:

- To measure the I-V characteristics of Zener diodes.

- To determine the voltage regulation for variable resistance and variable supply voltage.

Introduction:

The diodes we have studied before do not operate in the breakdown region because this may damage them. A Zener diode is different; it is a silicon diode that the manufacturer has optimized for operation in the breakdown region. It is used to build voltage regulator circuits that hold the load voltage almost constant despite large change in line voltage and load resistance. Figure shows the symbol of the Zener diode.

Cathode

Anode

Fig.1: Symbol of Zener diode

The Zener diode may have a breakdown voltage from about 2 to 200 V. These diodes can operate in any of the three regions: forward, leakage and breakdown. The figure shows the 1-V graph of the Zener diode.

Current

Forward

Current

Breakdown Voltage

Reverse Voltage

Leakage

Current

Voltage

Breakdown

Current

Fig. 2: I-V characteristic curve

- In the forward region it works as an ordinary diode.

- In the leakage region (between zero and breakdown) it has only a small reverse saturation current.

- In the breakdown it has a sharp knee, followed by an almost vertical increase in current

without changing the voltage.

- The voltage is almost constant, approximately equal to Vz over most of the breakdown

region.

Approximation:

“On”	[VVz	]					“Off” [Vz >V > 0]

Pre-Lab Report:

- Draw the I-V characteristic of Fig. 3 (circuit with no load )and

- calculate VL, VR, IZ for 10 V supply voltage.

- Consider RL = 220 Ω and write down the condition for conduction.

- Calculate RLmin, RLmax, ILmin for ILmax  for Izmax= 7.2mA and 10 V supply voltage in Fig. 4.

- Calculate Vi min, and Vi max for Izmax= 7.2mA of Fig. 5.

Equipments:

- Zener diode (6.2 volt)

Resistance (220 Ω, 470 Ω, 1 kΩ)

EMBED Visio.Drawing.11

470 Ω

6.2 V

EMBED Visio.Drawing.11

EMBED Visio.Drawing.11

220 Ω

VL

- POT 10 kΩ

- DC Power supply

- Bread board

- Multimeter.

- Ammeter.

Experimental Setup:

+ VR -

+ VR -

EMBED Visio.Drawing.11

Vi = 10V

Vi = 10V

6.2 V

470 Ω

Rv

Fig. 3: Circuit with No Load 	               Fig. 4: Circuit with variable load

+ VR -

EMBED Visio.Drawing.11

470 Ω

6.2 V

1KΩ

VL

EMBED Visio.Drawing.11

Vi

Fig. 5: Circuit with variable supply voltage.

Procedure:

- Connect the circuit as shown in Figure 3.

- Vary the supply voltage from zero to 10 volts and complete Table 1.

- Now, connect the circuit as shown in figure 4.

- Keep the POT (variable resistance Rv) at its maximum value which you have calculated from pre lab.

- Gradually decrease the POT resistance up to minimum Rv which you have calculated from pre lab , next calculate IL by ohm’s law.

- complete Table 2.

- Replace load with 1 kΩ resistance, vary the supply voltage Vi  and complete table 3.

| Vi (Volt) | VR (Volt) | Vz (Volt) | Iz = VR/R (mA) |
| --- | --- | --- | --- |
| 1 |  |  |  |
| 2 |  |  |  |
| 3 |  |  |  |
| 4 |  |  |  |
| 5 |  |  |  |
| 6 |  |  |  |
| 7 |  |  |  |
| 8 |  |  |  |
| 9 |  |  |  |
| 10 |  |  |  |

Table 1: Data for I-V characteristics

| Vi (Volt) =10V | VL(Volt) | IL(Amp) |
| --- | --- | --- |

Table 2: Data for regulation due to load variation

| Vi (Volt) | VL(Volt) |
| --- | --- |
| 1 |  |
| 2 |  |
| 3 |  |
| 4 |  |
| 5 |  |
| 6 |  |
| 7 |  |
| 8 |  |
| 9 |  |
| 10 |  |

Table 3: Data for regulation due to supply voltage variation

Post Lab Report Questions:

1. Plot the I-V characteristics of the Zener diode. Determine the Zener breakdown voltage from the plot.

2. Plot IL Vs VL for the data of table-2. Find the load regulation and compare it with the pre-lab data.

3. Plot VL Vs V for the data of table 3. Find the line regulation and compare it with the pre-lab data.