East West University

Lab Report

Semester: Summer-2025

Course Title: Electronic Circuits        Course Code: CSE251

Section: 04

Expt No:  04

Expt Name: Introduction to Transistor

Group No: 06

Group Members:

| Student Name | Student Id |
| --- | --- |
| Md. Hasib Ali | 2023-3-60-186 |
| Arijit Kumar Anoy | 2023-3-60-055 |
| Md. Shakib Al Hasan | 2024-1-60-200 |
| Fahim Shahriar | 2023-2-60-094 |
| Morshedul Islam | 2023-2-60-125 |

Submitted by-

Name: Md. Shakib Al Hasan

Id: 2024-1-60-200

Submitted to-

Dr. Sarwar Jahan 
Associate Professor 
Department of Computer Science & Engineering

East West University

Date of Performance: July 28, 2025

Date of Submission: August 09, 2025

Title: Introduction to Transistor

### Objectives:

1. Identify base, emitter and collector terminals and connections of NPN and PNP transistors.

2. Demonstrate and measure the effects on base current of forward and reverse bias in the emitter-base circuit.

Theory (Summary):

It is convenient to represent the current voltage characteristics of transistor graphically. In a BJT common-emitter (CE) configuration, the emitter serves as the common terminal between input and output. The input is applied at the base terminal and the output is taken from the collector terminal. The typical CE output describes ic as a function of VCE with is as a parameter. In the active mode, ic of practical BJTs shows some dependence on VCE due to early effect. As a result, ic characteristics are not horizontal straight lines. This dependency of icon VCE is included in the equivalent circuit via an output resistance ro.

Equipment and Component:

1. Power supply

2. Multimeter

3. Resistor (95.16ΚΩ and 1.8ΚΩ)

4. Transistor (NPN and PNP)

5. Voltmeter

6. DC milli-ammeter and DC micro-ammeter

Circuit Diagram:

Figure-1: Circuit diagram for measuring I-V characteristics of Transistor

Experimental Datasheet:

| VBB | VCC | VB | VC | VCE |
| --- | --- | --- | --- | --- |
| 10.119V | 1 | 9.510 | 0.9436 | 0.0085 |
|  | 2 | 9.502 | 1.8962 | 0.0146 |
|  | 3 | 9.495 | 2.912 | 0.201 |
|  | 4 | 9.491 | 3.959 | 0.0251 |
|  | 5 | 9.489 | 4.975 | 0.0296 |
|  | 6 | 9.487 | 5.929 | 0.0335 |
|  | 7 | 9.484 | 6.964 | 0.0374 |
|  | 8 | 9.483 | 8.007 | 0.0411 |
|  | 9 | 9.481 | 8.959 | 0.0442 |
|  | 10 | 9.480 | 9.988 | 0.0476 |

| VCC | VBB | VB | VC | VCE |
| --- | --- | --- | --- | --- |
| 10.011V | 1 | 0.3489 | 1.5382 | 8.476 |
|  | 2 | 1.3058 | 5.847 | 4.145 |
|  | 3 | 2.985 | 9.844 | 0.2058 |
|  | 4 | 3.302 | 9.938 | 0.0994 |
|  | 5 | 4.341 | 9.973 | 0.0802 |
|  | 6 | 5.317 | 9.966 | 0.0698 |
|  | 7 | 6.312 | 9.972 | 0.0624 |
|  | 8 | 7.301 | 9.982 | 0.0569 |
|  | 9 | 8.412 | 9.984 | 0.0519 |
|  | 10 | 9.393 | 10.00 | 0.0483 |

Post-Lab:

Plot Ic-VCE characteristics curve from the measured data using PSPICE and MATLAB

Answer

Discussion