East West University

Lab Report

Semester: Summer-2025

Course Title: Electronic Circuits        Course Code: CSE251

Section: 04

Expt No:  05

Expt Name: Adder Circuit using 741 Op Amp

Group No: 06

Group Members:

| Student Name | Student Id |
| --- | --- |
| Md. Hasib Ali | 2023-3-60-186 |
| Arijit Kumar Anoy | 2023-3-60-055 |
| Md. Shakib Al Hasan | 2024-1-60-200 |
| Fahim Shahriar | 2023-2-60-094 |
| Morshedul Islam | 2023-2-60-125 |

Submitted by-

Name: Md. Shakib Al Hasan

Id: 2024-1-60-200

Submitted to-

Dr. Sarwar Jahan 
Associate Professor 
Department of Computer Science & Engineering

East West University

Date of Performance: August 04, 2025

Date of Submission: August 11, 2025

Title: Adder Circuit using 741 Op Amp.

### Objectives

1. To familiarize with the 741 Op Amp Integrated Circuit (IC).

2. To design and construct an adder using 741 Op Amp.

Theory (Summary)

Operational Amplifier (Op Amp) is a differential amplifier and can perform mathematical operations such as addition, subtraction, etc. This is an integrated circuit (IC).

The block diagram of the 741 Op Amp is shown in Figure-1. It has 8 pins. To identify the pins of the 741 Op Amp in the laboratory, place the Op-Amp on the trainer board in such a way that the notch is in your left side. Then the pin number should be identified as shown in Figure-1.

Figure-1: Block diagram of the 741 Op-Amp

The names of different pins are as follows:

1: Offset null (usually not used)

2: Inverting input terminal

3: Non-inverting input terminal

4: Negative DC power supply (usually 5-15V negative)

5: Offset null (usually not used)

6: Output terminal

7: Positive DC power supply (usually 5-15V positive)

8: Not connected (NC)

The Op Amp acts as an adder when the non-inverting input terminal is grounded and two or more signals are fed to the inverting input via resistors of appropriate values. The output becomes negative of the sum of the input signals.

A non-inverting amplifier produces the amplified output equal to the input multiplied by the closed loop gain, which is determined by the appropriate resistances.

Equipment and Component

1. Digital trainer board

2. DC power supply

3. Signal generator

4. Oscilloscope

5. Digital multimeter

6. 741 Op Amp (1 pc)

7. Resistor (as required from pre-lab design)

8. Breadboard

9. Connecting wires

Circuit Diagram

Figure-2: an adder circuits using 741 Op Amp.

Pre-Lab

1. DESIGN OF ADDER CIRCUIT: Design the adder circuit shown in Figure-2 (determine the values of the resistors) so that it implements the function V=-(V1+2V2). The available set of resistances that you can use is 1002, 1ΚΩ, 2.2ΚΩ, 3.3ΚΩ, 5.6ΚΩ, and 10ΚΩ. For design safety, take Rf as high as possible. In your design, if exact value of resistance is not available, choose the nearest one.

Answer

As we are using only 1 Op Amp, we can design the circuit as follow:

We know, For adder circuit, Vo=- (RFV1R1+RFV2R2 +.……....+RFVNRN)……………. (i)

Comparing equation (i),

RFR1=1

RFR2=2

⸫ R2=0.5RF

Since we are required for taking RF  as high. RF =10KΩ, RF =5 KΩ.

Since we don’t have 5KΩ resistor, but we have 5.6KΩ.

⸫ R2≈5.6KΩ.

Post-Lab

1. From the measurement in step-3, verify your design.

Answer

Here, V1=5V; V2=2V;

Output voltage, Vo=-V1+2 V2V=- (5+4) V=- 9V

Vo=- (V1+2 V2) V=-(5+4) V=- 9V

And from the measurement Output voltage, V=-12.99V

2. Does the amplitude measured in step-4 verify your design? Explain.

Answer

Amplitude measured in step 4 is 4.80V as we used 5V from AC power supply and the output taken from DC mode. So, the amplitude is almost the same here.

3. Simulate the circuit shown in Figure-2 in Pspice. Use V1 a 5V peak to peak, 1KHZ, 00  phase sine wave and V2 a 5V peak to peak, 1KHZ, 900  phase sine wave. Perform simulation for 4 cycles (transient analysis for 4 ms) and attach the printed output with your report.

Answer

4. Write the expressions of V1 and V2 of Question-3 in phasor domain, add them, and write the result in time domain as a sine function. Compare it with the Pspice output in terms of amplitude, phase angle, and time period.

Answer

In Phasor domain,

Here, V1 =2.5∠00V , V2 = 2.5∠900 V

V1  + V2  = 2.5∠00V +2.5∠900 V= 3.536 ∠450V

So, phase angle, φ=450

Now,

Amplitude of magnitude, Vm=2.52 + 2.52=3.536 V

Angle, θ=tan-1(2.52.5)=450

In time domain we know,

Vt=Vmcos(ωt+φ)

Or,  Vt=Vm cos(2πft+φ)

V=3.536cos(2πft+450)      [ f=1kHz=1000Hz ]

=3.536cos(2000πt+450)

=3.536 sin(2000πt+450+900)

=-3.536sin(2000πt+1350)

We know,

Time period, T=1f=11000=10-3 s=1ms

From PSpice simulation,

We find amplitude = 4.80V, Phase angle = 900 And time period = 1ms

Comparison between PSpice and theoretical calculation:

|  | From PSpice | From calculation |
| --- | --- | --- |
| Amplitude | 4.8V | 4.8V |
| Phase angle | 900 | 450 |
| Time period | 1ms | 1ms |

Discussion