East West University

Lab Report

Semester: Summer-2025

Course Title: Electronic Circuits        Course Code: CSE251

Section: 04

Expt No:  06

Expt Name: Signal Integration and Differentiation using 741 Op Amp

Group No: 06

Group Members:

| Student Name | Student Id |
| --- | --- |
| Md. Hasib Ali | 2023-3-60-186 |
| Arijit Kumar Anoy | 2023-3-60-055 |
| Md. Shakib Al Hasan | 2024-1-60-200 |
| Fahim Shahriar | 2023-2-60-094 |
| Morshedul Islam | 2023-2-60-125 |

Submitted by-

Name: Md. Shakib Al Hasan

Id: 2024-1-60-200

Submitted to-

Dr. Sarwar Jahan 
Associate Professor 
Department of Computer Science & Engineering

East West University

Date of Performance: August 11, 2025

Date of Submission: August 18, 2025

Title: Signal Integration and Differentiation using 741 Op Amp

### Objectives

1. To study the response of 741 Op Amp Integrator to sinusoid and square waveforms.

2. To study the response of Op-Amp differentiators to sinusoid and triangular waveforms.

Theory (Summary)

An operational Amplifier (Op-Amp) is a differential amplifier that performs mathematical operations such as addition, subtraction, integration, differentiation, etc. In Expt. No. 3, we use the Op-Amp with the only resistors to study adder and amplifier circuits. In this experiment, we will study integrator and differentiator circuits by using a capacitor either in the feedback or input path.

Equipment and Component

1. Digital trainer board

2. Signal generator

3. Oscilloscope

4. Digital multimeter

5. 741 Op Amp (1 pc)

6. Resistor (1ΚΩ 1 pc)

7. Capacitor (0.1µF, 1 pc)

8. Breadboard

9. Connecting wires

Circuit Diagram

Figure-1: An Op Amp integrator circuits.

Figure-2: An Op Amp differential circuits

Pre-Lab

1. INTEGRATOR: Consider the integrator shown in Figure_1 with Vin = VPsin(ꞷt). If the circuit is initially relaxed, find the expression of output and determine the frequency, f, at which the output amplitude is equal to 2VP. Also, determine the phase relation between input and output.

Answer

Integrator Circuit Given

Input: Vin=VPsin(ꞷt)

Resistor: R = 1KΩ = 1000Ω

Capacitor: C=0.1µF=1×10-7F

Output Expression:

For an ideal op-amp integrator, the output is:

Voutt=-1Rc∫Vintdt

Substitute, Vint=VPsin(ꞷt)

Voutt=-VPRCωcos(ꞷt)

So, the output is a cosine wave with a phase shift of -900(lagging input by 900), and

amplitude Vout=VPRCω

Now when output amplitude = 2VP

⸫ VPRCω=2VP

Or, 2πf=12RC

Or, f=14πRC=14π×1×10-7×1000=795.7747Hz

Phase Relation; Output lags the input by 900

2. DIFFERENTIATOR: Consider the differentiator shown in Figure 2 with Vin=VPsin(ꞷt). in If the circuit is initially relaxed, find the expression of output and determine the frequency, f, at which the output amplitude is equal to V, 12. Also, determine the phase relation between input and output.

Answer

Differentiator Circuit Given

Input: Vin=VPsin(ꞷt)

Resistor: R = 1KΩ = 1000Ω

Capacitor: C=0.1µF=1×10-7F

Output Expression:

For an ideal op-amp integrator, the output is:

Voutt=-RCddtVint

Substitute, Vint=VPsin(ꞷt)

Voutt=-ꞷRCVpcos(ꞷt)

So, the output is a cosine wave with a phase-shifted -900(lagging input by 900), and

amplitude, Vout=-ꞷRCVp

Now when output amplitude =VP2

⸫ ꞷRCVp=VP2

Or, 2πf=12RC

Or, f=14πRC=14π×1×10-7×1000=795.7747Hz

Phase Relation: Output lags the input by 900

Post-Lab

INTEGRATOR:

1. In the output expression from your prelab, put the measured value of R and the given values of C and VP; calculate the amplitude of the output signal and compared it with the measured data.

Answer

Here,

R = 1KΩ = 1000Ω

C=0.1µF=1×10-7F

VP=2V

f = 1000Hz

We know,

Amplitude, Vo=VPωRC=VP2πfRC=12π×1000×1000×1×10-7=3.18V

Comparison between the measured & calculate the amplitude:

|  | Calculate Value | Measured Value | Discrepancy |
| --- | --- | --- | --- |
| Amplitude | 3.18 | 2.04 | 35.849% |

2. Compare the phase relation between your prelab result and measured data.

Answer

Here,

∆𝑡 = 260µ𝑠

f = 1000Hz

⸫  ∆θ=3600×∆t× f

=3600×260us×1000=93.6°

Comparison between the measured & prelab values of the phase difference:

|  | Prelab Value | Measured Value | Discrepancy |
| --- | --- | --- | --- |
| Phase Relation | 900 | 93.60 | 4% |

3. Simulate the integrator of Figure-1 for 4 cycles using PSpice with the measured value of R and the square wave input that you have used in the lab and have the simulation result printed. Compare the simulation result with your measurement.

Answer

Operation of Op Amp Integrator:

Integration of Sinusoidal Voltage:

Simulation Result:

Integration of Square wave Voltage:

Simulation Result:

Discussion