# East West University

Department of Computer Science and Engineering

Semester :  spring 2021

Course Title: Electrical Circuits

Course Code: CSE 209

Section: 03

Experiment No:  02

Experiment Title:  Series – Parallel  DC  Circuit and Verification of Kirchhoff’S  Laws

Student Name: Md. Rakibur Rahman

ID:  2019-3-60-113

Date of Performance: 22.03.2021

Date of Submission: 28.03.2021

Objectives:

- To learn analysis of dc series-parallel circuit.

- To verify Kirchhoff’s voltage law (KVL).

- To verify Kirchhoff’s current law (KCL).

Circuit Diagram:

| Measured Value of E(V) | Measured Value of V₁ (V) | Measured Value of V₂ (V) | Measured Value of V₃ (V) | Measured Value of I₁(mA) | Measured Value of I₂(mA) | Measured Value of I₃(mA) | Measured Value of Resistance(Ω) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 3 | 3 | 0.7268 | 0.7268 | 22.73 | 15.46 | 7.268 | R₁=100 Ω R₂=47Ω R₃=100 Ω |

Experimental Datasheet:

Answers to the post lab report questions:

- Here , R₂ and R₃ are parallel,

So, Equivalent resistance, Rp= 47×10047+100=31.973 Ω

Applying voltage divider rule,

V₁= ER1+Rp × R1

= 3100+31.973×100

= 2.273V

V₂ = ER1+Rp × Rp

= 3100+31.973×31.973

= 0.727V

Here , V₂ and V₃ are parallel.

So , V₂ = V₃ = 0.727V . Because the voltage is equal in Parallel Circuit .

From Ohm’s Law,

I₁=ER1+Rp

= 3100+31.973

= 0.02273A

= 22.73mA

Applying current divider rule,

I₂= I1R2+R3 × R3

= 22.7347+100×100

= 15.463mA

I₃= I1R2+R3 × R2

= 22.7347+100×47

= 7.267mA

Comparing the theoretically calculated values with the simulated values from PSpice.

|  | Calculated value | Measured value |
| --- | --- | --- |
| Current | I1 =22.73mA | I1 =22.73mA |
|  | I2= 15.463mA | I2 =15.46mA |
|  | I3= 7.267mA | I3= 7.268mA |
| Voltage | V1 = 2.273V | V1 = 3V |
|  | V2 = 0.727V = 727mV | V2 = 726.80mV |
|  | V3 = 0.727V = 727mV | V3 = 726.80mV |

The experimental values of the voltages and the currents are slightly different from the theoretical values.

- (i) From the calculated values we get,

V₂ = V₃ = 0.727V

Here, both V₂ and V₃ are parallel in the circuit so the

voltage are same.

(ii) From the calculated values we get,

V1 = 2.273V

V2 = 0.727V

Applying KVL,

V₁+V₂ = (2.273+0.727)V

= 3V

= E

(iii) From the calculated values we get,

I₁ = 22.73mA

I₂ = 15.463mA

I₃ = 7.267mA

Applying KCL,

I₂+ I₃ = (15.463+7.267)mA

= 22.73mA

= I₁

Conclusion:

From the experiment we learnt about the application Ohm’s Law, Kirchhoff’s Voltage Law(KVL) and Kirchhoff’s Current Law(KCL).