East West University

Department of Computer Science and Engineering

Semester : spring 2021

Course Title: Electrical Circuits

Course Code: CSE 209

Section: 03

Experiment No: 3

Experiment Title: Bias Point Detail Analysis of DC Circuit With Independent Sources Using PSpice Schematics.

Student Name: Md. Rakibur  Rahman

ID:  2019-3-60-113

Date of Performance: 10-3-2021

Date of Submission: 17-3-2021

Objectives:

1. To learn  fundamentals of PSpice.

2. To analyze  Bias Point Detail of DC circuit using PSpice Schematics.

Circuit Diagram:

Answers to the post lab report questions:

1)

Applying KCL at node 1,

V1=10 V

Applying KCL at node 2,

V2-V120 + V210 + V2-V330 = -3

V2-1020 + V210 + V2-V330 = -3

v2 ( 120 + 110 +130 ) + v3 (- 130 ) = -3+ 1020

.1833v2 - .0333v3 = -2.5 ……… (1)

Applying KCL at node 3,

V3-V230 + V310  = 3+10

v2 (- 130 ) + v3 ( 130 + 110 ) = 13

- .0333v2 + .1333v3 = 13…………. (2)

Solving equation (1) and (2) we get,

v2 = 4.272 V

v3= 98.592 V

Calculation of the current,

R1, I1= V1-V220 =.2864 A= 286.4mA

R2, I2= V3-V230=3.144 A

R3,I3 = V210 = 0.4272 A= 427.2mA

R4 ,I4 =V310 = 9.8592A

2. Comparing the theoretically calculated values with the simulated values from PSpice.

|  | Theoretical value | Obtained value |
| --- | --- | --- |
| Current | I1 =286.4 mA | I1 =285.71mA |
|  | I2= 3.144 A | I2 =3.143A |
|  | I3= 427.2 mA | I3= 428.57mA |
|  | I4= 9.8592A | I4= 9.86A |
| Voltage | V1=10V | V1=10V |
|  | V2=4.272V | V2= 4.286V |
|  | V3 =98.592V | V3 =98.57V |

The experimental values of the voltages and the currents are slightly different from the theoretical values.

Conclusion:

From this experiment we have learnt about analyzing bias point detail of a DC circuit, to calculate currents & voltages of a circuit using KCL. We have also learnt about a little discrepancy between the calculated & simulated values.