East West University

Department of Computer Science and Engineering

Semester :  Spring 2021

Course Title: Electrical Circuits

Course Code: CSE 209

Section: 03

Experiment Number : 06

Experiment Title :  Verification of Thevenin’s Theorem.

Student Name :  Md. Rakibur Rahman

ID: 2019-3-60-113

Date of Performance : 28 .04 .2021

Date of Submission : 04 .05 .2021

Objective : To verify the Thevenin’s theorem theoretically experimentally, and using  PSpice simulation.

Circuit Diagram:

Figure -1

Figure-2

Figure-3

Figure -4

Experimental Datasheet:

| Value of E1 | Value of E2 | Value of VL | Value of IL | Value of VOC | Value of ISC | Value of Resistors |
| --- | --- | --- | --- | --- | --- | --- |
| 10 | 5 | 3.260 | 3.260mA | 7.090 | 6.03mA | R₁=1 kΩ R₂=4.7 kΩ R₃=3.3 kΩ R4 = 1 kΩ RL = 1 kΩ |

Answers to the post lab report questions:

- For figure 1:

Applying KVL at mesh 1,

4.7ki1 + 1k(i1- i2) = -10

5.7ki1 - 1k i2 = -10……(1)

Applying KVL at mesh 2,

1k(i2- i1) + 1ki2 +  3.3k(i2- i3) = 10-5

-1ki1 + 5.3ki2 -  3.3k i3 = 5……(2)

Applying KVL at mesh 3,

3.3k(i3- i2)+ 1k i3=5

-3.3k i2+ 4.3k i3=5……(3)

Solving  equation 1,2 & 3,

i1=-1.275×10-3A =-1.275mA

i2=2.733×10-3A =2.733mA

i3= 3.260×10-3A= 3.260mA

here, i3=IL= 3.260mA

VL =IL× R= 3.260mA× 1kΩ

= 3.260V

For  figure 2:

Applying KCL at node 1,

V14.7k + V1-V21k + V1-101k = 0

2.213×10-3V1 - 1×10-3V2 =0.01 ……(1)

Applying KCL at node 2,

V2-V11k + V2-53.3k = 0

- 1×10-3V1 + 1.30303×10-3V2 =1.51515×10-3  ……(2)

Solving  equation 1&2,

v1= 7.723V

v2=7.090V

here, VOC=Vth = V2= 7.090V

For figure 3:

Applying KVL at mesh 1,

4.7ki1 + 1k(i1- i2) = -10

5.7ki1 - 1k i2 = -10……(1)

Applying KVL at mesh 2,

1k(i2- i1) + 1ki2 +  3.3k(i2- i3) = 10-5

-1ki1 + 5.3ki2 -  3.3k i3 = 5……(2)

Applying KVL at mesh 3,

3.3k(i3- i2)=5

0i1-3.3k i2+ 3.3k i3=5……(3)

Solving  equation 1,2 & 3,

i1=-9.6154×10-3A =-0.96154mA

i2=4.519×10-3A =4.519mA

i3= 6.034×10-3A= 6.034mA

here, i3=ISC= 6.034mA

Rth = VthIsc

=7.0906.034×10-3

=1175 Ω

=1.175 kΩ

And , Eth= Vth=VOC=7.090V

For figure 4:

Here , IL = 7.0901.175k+1k

= 3.260mA

VL =IL× R= 3.260mA× 1kΩ

= 3.260V

Thus, it verifies Thevenin’ theorem.

2. As the experiment was done in PSpice, the values of E ,V & R are same. Thus , there is no discrepancy.

Conclusion: In this experiment  we verified Thevenin’s equivalent theorem. Now we can convert two terminal linear circuits into a voltage source in series with a resistance as well as Thevenin’s equivalent circuit.