# East West University

Department of Computer Science and Engineering

Semester : Spring 2021

Course Title: Electrical Circuits

Course Code: CSE209

Section: 03

Experiment Number: 07

Experiment Title: DC Circuit Analysis in PSpice using Source and Resistance Sweep

Student Name: Md. Rakibur  Rahman

ID:  2019-3-60-113

Date of Performance: 05.05.2021

Date of Submission: 09.05.2021

Objectives:

- To analyze DC circuit in PSpice by sweeping source and resistance.

- To verify maximum power transfer theorem.

Circuit Diagram:

Answers to the post lab report questions:

- From step 4,

From step 5(d),

Comparing the values of V(1), V(2) and I(R3) obtained in steps 4 and 5(d),

| Values | Step 4 | Step 5(d) |
| --- | --- | --- |
| V1 | 4.4444V | 4.4444V |
| V2 | 2.2222V | 2.2222V |
| I(R3) | 222.222mA | 222.222mA |

Here, the values of step 4 & step 5(d) are same.

- From step 2,

Here , ISC =400mA =0.4A

Vth = 5V [from step 1]

Rth = VthIsc= 50.4 Ω = 12.5 Ω

Here , Rth =RL=12.5

From step 5(e),

Comparing the load resistance RL for maximum power transfer obtained in steps 2 and 5(e),

| Step 2 | Step 5(e) |
| --- | --- |
| 12.5 Ω | 12.5 Ω |

RL for maximum power transfer obtained in step 2 & 5(e) are same.

3.

Applying KCL at node 1,

V1-105 + V1-V210 + V15 = 0

0.5V1 – 0.1V2 =2…..(1)

Applying KCL at node 2,

V2-V110 + V210 = 0

-0.1V1 +0.2V2 =0……(2)

Solving  equation 1 & 2,

V1 =4.4444 V

V2 = 2.2222V

I(R3) =V1-V2R3

=4.4444-2.222210

=222.22mA

Comparing the theoretical solutions with the solutions obtained from PSpice,

| Values | PSpice | Calculated |
| --- | --- | --- |
| RL | 12.5 Ω | 12.5 Ω |
| V1 | 4.444V | 4.4444V |
| V2 | 2.222V | 2.2222V |
| I(R3) | 222.22mA | 222.22mA |

The calculated values & the simulated values are the same. Thus, there is no discrepancy.

Conclusion: In this experiment , we learnt about analyzing DC circuit in PSpice by sweeping source and resistance. And verifying maximum power transfer theorem. We found that, there is no discrepancy in the calculated & simulated values.