Physics Lab

Course Name  :  Engineering Physics-1

Course Code   :  PHY 109 LAB

Section  No     :

Group  No       :

Experiment No                :

Name of the Experiment:

Date of Allocation    :

Date of Submission :

Submitted To    :

Student’s ID      :

Student’s Name :

Physics Lab

## PHY-109 LAB

## Experiment No: 12

Name of the Experiment:

To determine the modulus of rigidity of a wire by the method of oscillations (Dynamic method).

Theory: Within the elastic limit of a body, the ratio of the shearing stress to shearing strain is called the modulus of rigidity. If a heavy body be supported by a vertical wire of length Ɩ and radius r, so that the axis of the wire passes through its center of gravity and if the body be turned through an angle and released, it will execute torsional oscillations about a vertical axis. If at any instant the angle of twist be  θ, the moment of the torsional couple exerted by the wire will be,

C   =      ηпr4θ2l

When  θ = 1 radian,

Then, C   =      ηпr42l  . . . . . . . . . . . . (1)

Where, η is the modulus of rigidity of the material of the wire.

Therefore, the motion is simple harmonic and of the fixed period.

T = 2пIC

Or, T2 = 4π2IC

Or, C =  4π2IT2  . . . . . .. . . .... (2)

Where,   I is the moment of inertia of the body.

And      	I   =     12 Ma2

Where, M = Mass of the cylinder, a = radius of the Cylinder.

From equation (1) and  (2) , we get

ηпr42l  = 4π2IT2

Or, η   = 4π2IT2  × 2lπr4

Or,  η =  8πIlT2r4    dynes/ sq.cm …………………..(3)

Putting the value of  I,l, r and T, we can calculate η.

Apparatus :

(i). A uniform wire 								        l

(ii). A disc or cylindrical bar

(iii). A suitable clamp

(iv). A stop-watch									m

(v). A screw gauge

(vi). A slide calipers

(vii). A meter scale

(viii). A balance, etc.

Procedure:

(i). We detached the cylinder from the suspension and weighed it with a balance. Also, we measured its diameter by means of a pair of slide calipers at three different places. Then we calculated the moment of inertia of the cylinder from its mass M and radius a using the relation, I =    12 Ma2 .

(ii). We measured the diameter of the wire by means of a screw gauge at three different points along the length of the wire, taking two mutually perpendicular readings at each position.

(iii). We suspended the cylinder with the experimental wire from the rigid support so that it rotates about the axis of the wire.

(iv). We measured the length of the wire from the point of support and the point at which the wire is attached to the cylinder with a rod and meter scale.

(v). We put a vertical chalk mark on the surface of the cylinder and when it is at rest, place a pointer facing the vertical line. In reference to this pointer, oscillations are counted. Alternately a telescope is to be focused from a distance on the vertical line on the cylinder so that it may remain coincident (without parallax) with the vertical line of the crosswire of the telescope.

(vi). We gave a little twist to the cylinder from its position of rest through a certain angle so that it begins to oscillate about its axis of suspension. With the help of a stopwatch, note the time for 20 complete oscillations. When the vertical line on the cylinder is going towards the right, crossing the tip of the pointer or the vertical line of the crosswire of the telescope, a stopwatch is started. The cylinder will perform one complete oscillation when the line on it crosses the pointer or the vertical line of the crosswire again in the same direction.

(vii). We repeated the operations three times and from these observations calculated the mean period of oscillation.

Data Collection:

(A) Length of the wire,  l =_____________ cm

Calculation of the least count:

Pitch (P) =   The distance moved by plane of the disc along vertical scale Number of full rotations given to the circular scale

=

=   ________________

Number of total divisions in the circular scale, n = 100

Least count (L.C.) 	=   pitchNumber of divisions of circular scale      =

=   ______________ mm

=   ______________ cm

(B) Reading for the Radius of the Wire (r)

| No. of obs. | Linear Scale Reading ( x ) cm | Circular Scale Divisions (CSD) | Least Count (LC) cm | Diameter d = x +(CSD × LC) cm | Mean Diameter (d) cm | Mean Radius r = d/2 cm |
| --- | --- | --- | --- | --- | --- | --- |
| 01 |  |  |  |  |  |  |
| 02 |  |  |  |  |  |  |
| 03 |  |  |  |  |  |  |

Vernier Constant =   One main scale divisionNumber of divisions in vernier scale  =

= ____________ mm

= ____________ cm

| No. of obs. | Main Scale Reading (x) cm | Vernier Scale Division (VSD) | Vernier Constant (VC) cm | Diameter D= x + (VSD × VC) cm | Mean Diameter D cm | Mean Radius a=D/2 cm |
| --- | --- | --- | --- | --- | --- | --- |
| 01 |  |  |  |  |  |  |
| 02 |  |  |  |  |  |  |
| 03 |  |  |  |  |  |  |

(C) Reading for the Radius of the Cylinder (a)

(D) Reading for the Time Period (T)

| No. of obs. | Time for 20 Oscillations (t) sec | Time Period T= (t/20) sec | Mean Time Period T sec |
| --- | --- | --- | --- |
| 01 |  |  |  |
| 02 |  |  |  |
| 03 |  |  |  |
| 04 |  |  |  |
| 05 |  |  |  |

Calculation:

Mass of the cylinder, M = ____________________ gm

Moment of inertia of the cylinder, 	I =    12 M a2

= _______________________  gm-cm2

= _______________________ gm-cm2

From data collection,

l = _________ cm, r = _________ cm, T = _________ sec, I =  ______________ gm-cm2

So, we get from equation (3)

η =   8πIlT2r4

= 	                                                                                      dynes/ sq.cm

=	                                                            dynes/ sq.cm

Standard value (Theoretical value) of η = 7.7 x 1011  dynes/ sq.cm  (Iron wire)

Percentage difference =   _____________

Result:

The Modulus of rigidity of a wire is= _________________ dynes/ sq.cm

Precautions & Discussions:

- There should be no knot in the wire.

- The amplitude of oscillation should be small so that wire is not twisted beyond the elastic limit.

- To avoid the backlash error, the circular scale reading of screw gauge should be taken very carefully.

- The radius of the suspension wire occurs in the 4th power and hence it should be measured very carefully.

Sample Oral Questions and Answers

1. What is modulus of rigidity?

Ans: The modulus of rigidity of an object is a measure of the resistance to deformation of an object when an external force is applied. It is expressed as the ratio of shear stress to shear strain.

2. What are the factors affecting modulus of rigidity?

Ans: The factors that affect modulus of rigidity are the material properties and the applicable force over the material.

3. What is an oscillation?

Ans: The repeated back and forth movement between two positions or states of an object is known as oscillation. The motion of the body is said to be oscillatory or vibratory motion if it moves back and forth about a fixed position or point after a regular interval of time.

4. What is moment of inertia?

Ans: Moment of Inertia is a quantity which expresses the body's tendency to resist external angular acceleration. It can be calculated as the sum of the products of the mass of each particle of the body with the square of its distance from the axis along which it is to be rotated.

5. What are the types of elastic constants?

Ans: Various types of elastic constants are:

- Bulk modulus

- Young’s modulus or modulus of elasticity

- Poisson’s ratio

- Shear modulus or modulus of rigidity

6. What are the different types of stress?

Ans. The different types of stress include:

- Tensile stress: Stress that results from pulling or stretching forces.

- Compressive stress: Stress that results from compressing or squeezing forces.

- Shear stress: Stress that results from forces acting parallel to each other but in opposite directions, causing deformation or sliding.